SRSP-321.2

Issue 1

January 2025

Spectrum Management

Standard Radio System Plan

# Technical Requirements for the Fixed Line-of-Sight Radio Systems Operating in the Band 21.2–23.6 GHz

Aussi disponible en français - PNRH-321.2

Preface

Standard Radio System Plan SRSP-321.2, issue 1, Technical Requirements for Fixed Line-of-Sight Radio Systems Operating in the Band 21.2–23.6 GHz replaces Technical Requirements for Fixed Line-of-Sight Radio Systems Operating in the Bands 21.8–22.4 GHz and 23.0–23.6 GHz, issue 1.

The following are the main changes:

- The frequency band covered by this SRSP has been expanded from 21.8–22.4 GHz and 23.0–23.6 GHz to the full 21.2–23.6 GHz range, consistent with recent spectrum policy decisions.

- The radio frequency (RF) channel arrangement has been updated to include the extended band segments 21.2–21.8 GHz and 22.4–23.0 GHz.

- The antenna radiation pattern requirements prescribed by envelope A and envelope B were modified in section 7.

- The radio frequency (RF) channel arrangement was updated to add RF channel bandwidths of 60 MHz, 80 MHz and 100 MHz.

- Redundant requirements already covered by SRSP-300-Gen were removed.

- Other editorial changes and clarifications have been made throughout the document.

Issued under the authority of

the Minister of Innovation, Science and Industry

____________________________________

Wen Kwan

Director General

Engineering, Planning and Standards Branch

# Intent

  - This Standard Radio System Plan (SRSP) states the minimum technical requirements for the efficient use of the frequency band 21.2–23.6 GHz by point-to-point radio systems in the fixed service.

  - This SRSP is intended to be employed in the design and specification of radio systems and equipment and in the evaluation of technical applications for new radio facilities or modification to radio systems submitted in accordance with the current issue of Radio Standards Procedure (RSP) 113, Application Procedures for Planned Radio Stations Above 960 MHz in the Fixed Service.

  - This SRSP must be used in conjunction with SRSP-300-Gen, General Technical Requirements for Fixed Radio Systems Operating in Frequency Bands above 960 MHz, for assessing compliance with Innovation, Science and Economic Development Canada (ISED) requirements. Certain technical requirements which were found in the previous issue of SRSP-321.8 are now addressed in SRSP-300-Gen. Fixed point-to-point radio systems in the frequency band 21.2-23.6 GHz shall comply with both the requirements of this SRSP as well as those prescribed in SRSP-300-Gen. Where requirements in SRSP-300-Gen are different from those in this SRSP, this SRSP shall take precedence.

  - This SRSP specifies equipment characteristics relating to efficient spectrum usage only and is not to be regarded as a comprehensive specification for equipment design and/or selection.

# General

  - This Standard replaces SRSP-321.8, Issue 1. Revision of this SRSP will be made as required.

  - Existing radio systems that were licensed as standard prior to the publication of this SRSP may continue to operate as standard. Extension, expansion, or modification of these systems will be considered by ISED on a case-by-case basis. New systems shall conform to the requirements of this standard.

# Related documents

The current issues of the following documents are applicable and available on the Spectrum Management and Telecommunications website:

| SRSP-300-Gen | General Technical Requirements for Fixed Radio Systems Operating in Frequency Bands above 960 MHz |
| --- | --- |
| SMSE-013-25 | Decision on the Policy, Technical and Licensing Framework for the Use of the Frequency Bands 21.2 – 21.8 GHz and 22.4 – 23.0 GHz by Fixed Services |
| RSP-113 | Application Procedures for Planned Radio Stations Above 960 MHz in the Fixed Service |
| TRC-43 | Designation of Emissions, Class of Station and Nature of Service |
| CTFA | Canadian Table of Frequency Allocations |
| CPC-2-0-03 | Radiocommunication and Broadcasting Antenna Systems |

CPC – Client Procedures Circular

RSP – Radio Standards Procedure

SRSP – Standard Radio System Plan

SP – Spectrum Utilization Policy

TRC – Telecommunications Regulation Circular

# Radio frequency channel arrangement description

The radio frequency channel arrangements defined in this Standard provide for 11 different RF channel spacings. Channel pairs are provided with a common transmit/receive separation of 1200 MHz. These arrangements apply across the entire frequency range 21.2–23.6 GHz, comprising a lower block (21.2–22.4 GHz) and an upper block (22.4–23.6 GHz). The standard supports the following RF channel spacings: 10, 15, 20, 40, 50, 60, 80 and 100 MHz. Moreover, the tables in annex A provides the RF channel frequencies.

  - 100 MHz channel spacing

The centre frequencies of the 12 paired channels that allow RF channel bandwidths greater than 80 MHz and less than or equal to 100 MHz and a transmit/receive separation of 1200 MHz are expressed by the following relationships:

In=22450-100n MHz, for n=1 to 12

In'=23650-100n MHz, for n=1 to 12

where n is the channel number and In and I'n are the centre frequencies in MHz of the paired channels in the lower half of the band and upper half of the band respectively.

  - 80 MHz channel spacing

The centre frequencies of the 15 paired channels that allow RF channel bandwidths greater than 60 MHz and less than or equal to 80 MHz and a transmit/receive separation of 1200 MHz are expressed by the following relationships:

Jn=22440-80n MHz, for n=1 to 15

Jn'=23640-80n MHz, for n=1 to 15

where n is the channel number and Jn and J'n are the centre frequencies in MHz of the paired channels in the lower half of the band and upper half of the band respectively.

  - 60 MHz channel spacing

The centre frequencies of the 20 paired channels that allow RF channel bandwidths greater than 50 MHz and less than or equal to 60 MHz and a transmit/receive separation of 1200 MHz are expressed by the following relationships:

Kn=22430-60n MHz, for n=1 to 20

Kn'=23630-60n MHz, for n=1 to 20

where n is the channel number and Kn and K'n are the centre frequencies in MHz of the paired channels in the lower half of the band and upper half of the band respectively.

  - 50 MHz channel spacing

The centre frequencies of the 24 paired channels that allow RF channel bandwidths greater than 40 MHz and less than or equal to 50 MHz and a transmit/receive separation of 1200 MHz are expressed by the following relationships:

An=21775+50n MHz, for n=1 to 12

An'=22975+50n MHz, for n=1 to 12

An=21225+50n-13 MHz, for n=13 to 24

An'=22425+50n-13 MHz, for n=13 to 24

where n is the channel number and An and A'n are the centre frequencies in MHz of the paired channels in the lower half of the band and upper half of the band respectively.

  - 40 MHz channel spacing

The centre frequencies of the 30 paired channels that allow RF channel bandwidths greater than 20 MHz and less than or equal to 40 MHz and a transmit/receive separation of 1200 MHz are expressed by the following relationships:

Bn=21780+40n MHz, for n=1 to 15

Bn'=22980+40n MHz, for n=1 to 15

Bn=21220+40n-16 MHz, for n=16 to 30

Bn'=22420+40n-16 MHz, for n=16 to 30

where n is the channel number and Bn and B'n are the centre frequencies in MHz of the paired channels in the lower half of the band and upper half of the band respectively.

  - 20 MHz channel spacing

The centre frequencies of the 60 paired channels that allow RF channel bandwidths greater than 15 MHz and less than or equal to 20 MHz and a transmit/receive separation of 1200 MHz are expressed by the following relationships:

Cn=22410-20n MHz, for n=1 to 60

Cn'=23610-20n MHz, for n=1 to 60

where n is the channel number and Cn and C'n are the centre frequencies in MHz of the paired channels in the lower half of the band and upper half of the band respectively.

  - 15 MHz channel spacing

The centre frequencies of the 80 paired channels that allow RF channel bandwidths greater than 10 MHz and less than or equal to 15 MHz and a transmit/receive separation of 1200 MHz are expressed by the following relationships:

Dn=22407.5-15n MHz, for n=1 to 80

Dn'=23607.5-15n MHz, for n=1 to 80

where n is the channel number and Dn and D'n are the centre frequencies in MHz of the paired channels in the lower half of the band and upper half of the band respectively.

  - 10 MHz channel spacing

The centre frequencies of the 120 paired channels that allow RF channel bandwidths less than or equal to 10 MHz and a transmit/receive separation of 1200 MHz are expressed by the following relationships:

En=22405-10n MHz, for n=1 to 120

En'=23605-10n MHz, for n=1 to 120

where n is the channel number and En and E'n are the centre frequencies in MHz of the paired channels in the lower half of the band and upper half of the band respectively.

  - Closed loops

Systems must be designed so that any closed loop will consist of an even number of hops.

  - Assignment of frequencies

New systems using any of the 100 MHz, 80 MHz, 60 MHz, 50 MHz or 40 MHz channel plans should use the lowest available frequency pair that can be successfully coordinated.

New systems using any of the 20 MHz, 15 MHz, or 10 MHz channel plans should use the highest available frequency pair starting from 22.4 GHz and 23.6 GHz that can be successfully coordinated.

Figure 1: 21.2-22.4 GHz and 22.4-23.6 GHz growth plan.

[Description of figure: Figure 1 Illustrates the growth plans, which indicate the order in which channels of various sizes will normally be assigned. These growth plans are also described in Section 4.10]

For multi-channel systems, channels should be chosen to minimize the number of wider bandwidth channels that may be blocked.

The use of co-channel dual polarization (CCDP) is both permitted and encouraged by ISED as a measure of increasing backhaul capacity while minimizing the need for additional spectrum. An applicant requesting paired channels using an RF channel of 60 MHz or 80 MHz or 100 MHz, and not employing CCDP technology, could be required to provide technical justification for not employing CCDP on a smaller RF channel spacing.

[Editor’s note: ISED is seeking feedback on whether applicants requesting paired channels with RF channel bandwidths of 60, 80 or 100 MHz, and not employing co-channel dual polarization (CCDP), should be required to provide technical justification for not using CCDP with a smaller RF channel spacing.]

  - Spectral efficiency

Digital systems submitted for licensing shall have a minimum spectral efficiency of 0.8 bit/s/Hz on a single polarization in a bandwidth corresponding to the channel spacing.

[Editor’s note: ISED is seeking feedback on the appropriateness of increasing the minimum spectral efficiency requirement for digital systems to 2.4 bit/s/Hz on a single polarization in a bandwidth corresponding to the channel spacing.]

# Transmitter characteristics

  - The transmitter power delivered to the antenna input shall not exceed 10 Watts (+10 dBW) per RF channel.

  - The centre frequency of the emission shall be maintained within ±0.001% of the assigned frequency.

# Maximum equivalent isotropically radiated power

The maximum equivalent isotropically radiated power (e.i.r.p.) from the antenna shall not exceed +55 dBW per RF channel.

# Antenna characteristics

In moderately congested and highly congested areas, the co-polarized radiation pattern envelope in the horizontal plane of the antenna must remain within Envelope A defined in Table 1 and shown in Figure 2, for both vertical and horizontal polarization.

Outside of moderately congested and highly congested areas, the co-polarized radiation pattern envelope in the horizontal plane of the antenna must remain within the envelope B defined in Table 2 and shown in Figure 2 for both vertical and horizontal polarizations.

The criteria for determining congestion levels are described in SRSP-300-Gen, General Technical Requirements for Fixed Radio Systems Operating in Frequency Bands above 960 MHz.

Table 1 : Minimum antenna characteristics for point-to-point digital radio systems operating in the band 21.2- 23.6 GHz – Envelope A

| Azimuth in degrees from main lobe | Antenna directivity in dB down from main lobe |
| --- | --- |
| 0 to 5 | 0 |
| 5 to 10 | 18 |
| 10 to 15 | 26 |
| 15 to 20 | 26 |
| 20 to 30 | 33 |
| 30 to 100 | 33 |
| 100 to 140 | 55 |
| 140 to 180 | 55 |

Table 2 : Minimum antenna characteristics for point-to-point digital radio systems operating in the band 21.2- 23.6 GHz – Envelope B

| Azimuth in degrees from main lobe | Antenna directivity in dB down from main lobe |
| --- | --- |
| 0 to 5 | 0 |
| 5 to 10 | 17 |
| 10 to 15 | 24 |
| 15 to 20 | 24 |
| 20 to 30 | 29 |
| 30 to 100 | 29 |
| 100 to 140 | 40 |
| 140 to 180 | 50 |

Figure 2: Minimum antenna characteristics for point-to-point digital radio systems operating in the band 21.2-23.6 GHz

[Description of figure: This line graph shows the antenna radiation pattern limits for point-to-point digital radio systems operating in the band 21.2-23.6 GHz. The x-axis plots azimuth in degrees from the main lobe from 0 to 180. The y-axis measures antenna directivity in dB from the main lobe from 0 to -80.

There are two data lines. The data line for Envelope A ranges from 0 to -55 on the y-axis. Meanwhile the data line for Envelope B ranges from 0 to -50 on the y-axis. There is a scale change indicator at the 20 mark on the x-axis. The measured radiation pattern in the horizontal plane must be within these limits for vertical and horizontal polarizations. Several data points for the lines are shown in tables 1 and 2.]

Annex A: RF channel centre frequencies

The channel arrangement center frequencies specified in section 4 of this SRSP are listed in the tables below.

Table A.1: Centre frequencies for 100 MHz channels

| Channel Pair | Lower Channel Centre Frequency (MHz) | Upper Channel Centre Frequency (MHz) |
| --- | --- | --- |
| I1 / I′1 | 22350 | 23550 |
| I2 / I′2 | 22250 | 23450 |
| I3 / I′3 | 22150 | 23350 |
| I4 / I′4 | 22050 | 23250 |
| I5 / I′5 | 21950 | 23150 |
| I6 / I′6 | 21850 | 23050 |
| I7 / I′7 | 21750 | 22950 |
| I8 / I′8 | 21650 | 22850 |
| I9 / I′9 | 21550 | 22750 |
| I10 / I′10 | 21450 | 22650 |
| I11 / I′11 | 21350 | 22550 |
| I12 / I′12 | 21250 | 22450 |

Table A.2: Centre frequencies for 80 MHz channels

| Channel Pair | Lower Channel Centre Frequency (MHz) | Upper Channel Centre Frequency (MHz) |
| --- | --- | --- |
| J1 / J′1 | 22360 | 23560 |
| J2 / J′2 | 22280 | 23480 |
| J3 / J′3 | 22200 | 23400 |
| J4 / J′4 | 22120 | 23320 |
| J5 / J′5 | 22040 | 23240 |
| J6 / J′6 | 21960 | 23160 |
| J7 / J′7 | 21880 | 23080 |
| J8 / J′8 | 21800 | 23000 |
| J9 / J′9 | 21720 | 22920 |
| J10 / J′10 | 21640 | 22840 |
| J11 / J′11 | 21560 | 22760 |
| J12 / J′12 | 21480 | 22680 |
| J13 / J′13 | 21400 | 22600 |
| J14 / J′14 | 21320 | 22520 |
| J15 / J′15 | 21240 | 22440 |

Table A.3: Centre frequencies for 60 MHz channels

| Channel Pair | Lower Channel Centre Frequency (MHz) | Upper Channel Centre Frequency (MHz) |
| --- | --- | --- |
| K1 / K′1 | 22370 | 23570 |
| K2 / K′2 | 22310 | 23510 |
| K3 / K′3 | 22250 | 23450 |
| K4 / K′4 | 22190 | 23390 |
| K5 / K′5 | 22130 | 23330 |
| K6 / K′6 | 22070 | 23270 |
| K7 / K′7 | 22010 | 23210 |
| K8 / K′8 | 21950 | 23150 |
| K9 / K′9 | 21890 | 23090 |
| K10 / K′10 | 21830 | 23030 |
| K11 / K′11 | 21770 | 22970 |
| K12 / K′12 | 21710 | 22910 |
| K13 / K′13 | 21650 | 22850 |
| K14 / K′14 | 21590 | 22790 |
| K15 / K′15 | 21530 | 22730 |
| K16 / K′16 | 21470 | 22670 |
| K17 / K′17 | 21410 | 22610 |
| K18 / K′18 | 21350 | 22550 |
| K19 / K′19 | 21290 | 22490 |
| K20 / K′20 | 21230 | 22430 |

Table A.4: Centre frequencies for 50 MHz channels

| Channel Pair | Lower Channel Centre Frequency (MHz) | Upper Channel Centre Frequency (MHz) |
| --- | --- | --- |
| A1 / A′1 | 21825 | 23025 |
| A2 / A′2 | 21875 | 23075 |
| A3 / A′3 | 21925 | 23125 |
| A4 / A′4 | 21975 | 23175 |
| A5 / A′5 | 22025 | 23225 |
| A6 / A′6 | 22075 | 23275 |
| A7 / A′7 | 22125 | 23325 |
| A8 / A′8 | 22175 | 23375 |
| A9 / A′9 | 22225 | 23425 |
| A10 / A′10 | 22275 | 23475 |
| A11 / A′11 | 22325 | 23525 |
| A12 / A′12 | 22375 | 23575 |
| A13 / A′13 | 21225 | 22425 |
| A14 / A′14 | 21275 | 22475 |
| A15 / A′15 | 21325 | 22525 |
| A16 / A′16 | 21375 | 22575 |
| A17 / A′17 | 21425 | 22625 |
| A18 / A′18 | 21475 | 22675 |
| A19 / A′19 | 21525 | 22725 |
| A20 / A′20 | 21575 | 22775 |
| A21 / A′21 | 21625 | 22825 |
| A22 / A′22 | 21675 | 22875 |
| A23 / A′23 | 21725 | 22925 |
| A24 / A′24 | 21775 | 22975 |

Table A.5: Centre frequencies for 40 MHz channels

| Channel Pair | Lower Channel Centre Frequency (MHz) | Upper Channel Centre Frequency (MHz) |
| --- | --- | --- |
| B1 / B′1 | 21820 | 23020 |
| B2 / B′2 | 21860 | 23060 |
| B3 / B′3 | 21900 | 23100 |
| B4 / B′4 | 21940 | 23140 |
| B5 / B′5 | 21980 | 23180 |
| B6 / B′6 | 22020 | 23220 |
| B7 / B′7 | 22060 | 23260 |
| B8 / B′8 | 22100 | 23300 |
| B9 / B′9 | 22140 | 23340 |
| B10 / B′10 | 22180 | 23380 |
| B11 / B′11 | 22220 | 23420 |
| B12 / B′12 | 22260 | 23460 |
| B13 / B′13 | 22300 | 23500 |
| B14 / B′14 | 22340 | 23540 |
| B15 / B′15 | 22380 | 23580 |
| B16 / B′16 | 21220 | 22420 |
| B17 / B′17 | 21260 | 22460 |
| B18 / B′18 | 21300 | 22500 |
| B19 / B′19 | 21340 | 22540 |
| B20 / B′20 | 21380 | 22580 |
| B21 / B′21 | 21420 | 22620 |
| B22 / B′22 | 21460 | 22660 |
| B23 / B′23 | 21500 | 22700 |
| B24 / B′24 | 21540 | 22740 |
| B25 / B′25 | 21580 | 22780 |
| B26 / B′26 | 21620 | 22820 |
| B27 / B′27 | 21660 | 22860 |
| B28 / B′28 | 21700 | 22900 |
| B29 / B′29 | 21740 | 22940 |
| B30 / B′30 | 21780 | 22980 |

Table A.6: Centre frequencies for 20 MHz channels

| Channel Pair | Lower Channel Centre Frequency (MHz) | Upper Channel Centre Frequency (MHz) |
| --- | --- | --- |
| C1 / C′1 | 22390 | 23590 |
| C2 / C′2 | 22370 | 23570 |
| C3 / C′3 | 22350 | 23550 |
| C4 / C′4 | 22330 | 23530 |
| C5 / C′5 | 22310 | 23510 |
| C6 / C′6 | 22290 | 23490 |
| C7 / C′7 | 22270 | 23470 |
| C8 / C′8 | 22250 | 23450 |
| C9 / C′9 | 22230 | 23430 |
| C10 / C′10 | 22210 | 23410 |
| C11 / C′11 | 22190 | 23390 |
| C12 / C′12 | 22170 | 23370 |
| C13 / C′13 | 22150 | 23350 |
| C14 / C′14 | 22130 | 23330 |
| C15 / C′15 | 22110 | 23310 |
| C16 / C′16 | 22090 | 23290 |
| C17 / C′17 | 22070 | 23270 |
| C18 / C′18 | 22050 | 23250 |
| C19 / C′19 | 22030 | 23230 |
| C20 / C′20 | 22010 | 23210 |
| C21 / C′21 | 21990 | 23190 |
| C22 / C′22 | 21970 | 23170 |
| C23 / C′23 | 21950 | 23150 |
| C24 / C′24 | 21930 | 23130 |
| C25 / C′25 | 21910 | 23110 |
| C26 / C′26 | 21890 | 23090 |
| C27 / C′27 | 21870 | 23070 |
| C28 / C′28 | 21850 | 23050 |
| C29 / C′29 | 21830 | 23030 |
| C30 / C′30 | 21810 | 23010 |
| C31 / C′31 | 21790 | 22990 |
| C32 / C′32 | 21770 | 22970 |
| C33 / C′33 | 21750 | 22950 |
| C34 / C′34 | 21730 | 22930 |
| C35 / C′35 | 21710 | 22910 |
| C36 / C′36 | 21690 | 22890 |
| C37 / C′37 | 21670 | 22870 |
| C38 / C′38 | 21650 | 22850 |
| C39 / C′39 | 21630 | 22830 |
| C40 / C′40 | 21610 | 22810 |
| C41 / C′41 | 21590 | 22790 |
| C42 / C′42 | 21570 | 22770 |
| C43 / C′43 | 21550 | 22750 |
| C44 / C′44 | 21530 | 22730 |
| C45 / C′45 | 21510 | 22710 |
| C46 / C′46 | 21490 | 22690 |
| C47 / C′47 | 21470 | 22670 |
| C48 / C′48 | 21450 | 22650 |
| C49 / C′49 | 21430 | 22630 |
| C50 / C′50 | 21410 | 22610 |
| C51 / C′51 | 21390 | 22590 |
| C52 / C′52 | 21370 | 22570 |
| C53 / C′53 | 21350 | 22550 |
| C54 / C′54 | 21330 | 22530 |
| C55 / C′55 | 21310 | 22510 |
| C56 / C′56 | 21290 | 22490 |
| C57 / C′57 | 21270 | 22470 |
| C58 / C′58 | 21250 | 22450 |
| C59 / C′59 | 21230 | 22430 |
| C60 / C′60 | 21210 | 22410 |

Table A.7: Centre frequencies for 15 MHz channels

| Channel Pair | Lower Channel Centre Frequency (MHz) | Upper Channel Centre Frequency (MHz) | Channel Pair | Lower Channel Centre Frequency (MHz) | Upper Channel Centre Frequency (MHz) |
| --- | --- | --- | --- | --- | --- |
| D1 / D′1 | 22392.5 | 23592.5 | D41 / D′41 | 21792.5 | 22992.5 |
| D2 / D′2 | 22377.5 | 23577.5 | D42 / D′42 | 21777.5 | 22977.5 |
| D3 / D′3 | 22362.5 | 23562.5 | D43 / D′43 | 21762.5 | 22962.5 |
| D4 / D′4 | 22347.5 | 23547.5 | D44 / D′44 | 21747.5 | 22947.5 |
| D5 / D′5 | 22332.5 | 23532.5 | D45 / D′45 | 21732.5 | 22932.5 |
| D6 / D′6 | 22317.5 | 23517.5 | D46 / D′46 | 21717.5 | 22917.5 |
| D7 / D′7 | 22302.5 | 23502.5 | D47 / D′47 | 21702.5 | 22902.5 |
| D8 / D′8 | 22287.5 | 23487.5 | D48 / D′48 | 21687.5 | 22887.5 |
| D9 / D′9 | 22272.5 | 23472.5 | D49 / D′49 | 21672.5 | 22872.5 |
| D10 / D′10 | 22257.5 | 23457.5 | D50 / D′50 | 21657.5 | 22857.5 |
| D11 / D′11 | 22242.5 | 23442.5 | D51 / D′51 | 21642.5 | 22842.5 |
| D12 / D′12 | 22227.5 | 23427.5 | D52 / D′52 | 21627.5 | 22827.5 |
| D13 / D′13 | 22212.5 | 23412.5 | D53 / D′53 | 21612.5 | 22812.5 |
| D14 / D′14 | 22197.5 | 23397.5 | D54 / D′54 | 21597.5 | 22797.5 |
| D15 / D′15 | 22182.5 | 23382.5 | D55 / D′55 | 21582.5 | 22782.5 |
| D16 / D′16 | 22167.5 | 23367.5 | D56 / D′56 | 21567.5 | 22767.5 |
| D17 / D′17 | 22152.5 | 23352.5 | D57 / D′57 | 21552.5 | 22752.5 |
| D18 / D′18 | 22137.5 | 23337.5 | D58 / D′58 | 21537.5 | 22737.5 |
| D19 / D′19 | 22122.5 | 23322.5 | D59 / D′59 | 21522.5 | 22722.5 |
| D20 / D′20 | 22107.5 | 23307.5 | D60 / D′60 | 21507.5 | 22707.5 |
| D21 / D′21 | 22092.5 | 23292.5 | D61 / D′61 | 21492.5 | 22692.5 |
| D22 / D′22 | 22077.5 | 23277.5 | D62 / D′62 | 21477.5 | 22677.5 |
| D23 / D′23 | 22062.5 | 23262.5 | D63 / D′63 | 21462.5 | 22662.5 |
| D24 / D′24 | 22047.5 | 23247.5 | D64 / D′64 | 21447.5 | 22647.5 |
| D25 / D′25 | 22032.5 | 23232.5 | D65 / D′65 | 21432.5 | 22632.5 |
| D26 / D′26 | 22017.5 | 23217.5 | D66 / D′66 | 21417.5 | 22617.5 |
| D27 / D′27 | 22002.5 | 23202.5 | D67 / D′67 | 21402.5 | 22602.5 |
| D28 / D′28 | 21987.5 | 23187.5 | D68 / D′68 | 21387.5 | 22587.5 |
| D29 / D′29 | 21972.5 | 23172.5 | D69 / D′69 | 21372.5 | 22572.5 |
| D30 / D′30 | 21957.5 | 23157.5 | D70 / D′70 | 21357.5 | 22557.5 |
| D31 / D′31 | 21942.5 | 23142.5 | D71 / D′71 | 21342.5 | 22542.5 |
| D32 / D′32 | 21927.5 | 23127.5 | D72 / D′72 | 21327.5 | 22527.5 |
| D33 / D′33 | 21912.5 | 23112.5 | D73 / D′73 | 21312.5 | 22512.5 |
| D34 / D′34 | 21897.5 | 23097.5 | D74 / D′74 | 21297.5 | 22497.5 |
| D35 / D′35 | 21882.5 | 23082.5 | D75 / D′75 | 21282.5 | 22482.5 |
| D36 / D′36 | 21867.5 | 23067.5 | D76 / D′76 | 21267.5 | 22467.5 |
| D37 / D′37 | 21852.5 | 23052.5 | D77 / D′77 | 21252.5 | 22452.5 |
| D38 / D′38 | 21837.5 | 23037.5 | D78 / D′78 | 21237.5 | 22437.5 |
| D39 / D′39 | 21822.5 | 23022.5 | D79 / D′79 | 21222.5 | 22422.5 |
| D40 / D′40 | 21807.5 | 23007.5 | D80 / D′80 | 21207.5 | 22407.5 |

Table A.8: Centre frequencies for 10 MHz channels

| Channel Pair | Lower Channel Centre Frequency (MHz) | Upper Channel Centre Frequency (MHz) | Channel Pair | Lower Channel Centre Frequency (MHz) | Upper Channel Centre Frequency (MHz) |
| --- | --- | --- | --- | --- | --- |
| E1 / E′1 | 22395 | 23595 | E61 / E′61 | 21795 | 22995 |
| E2 / E′2 | 22385 | 23585 | E62 / E′62 | 21785 | 22985 |
| E3 / E′3 | 22375 | 23575 | E63 / E′63 | 21775 | 22975 |
| E4 / E′4 | 22365 | 23565 | E64 / E′64 | 21765 | 22965 |
| E5 / E′5 | 22355 | 23555 | E65 / E′65 | 21755 | 22955 |
| E6 / E′6 | 22345 | 23545 | E66 / E′66 | 21745 | 22945 |
| E7 / E′7 | 22335 | 23535 | E67 / E′67 | 21735 | 22935 |
| E8 / E′8 | 22325 | 23525 | E68 / E′68 | 21725 | 22925 |
| E9 / E′9 | 22315 | 23515 | E69 / E′69 | 21715 | 22915 |
| E10 / E′10 | 22305 | 23505 | E70 / E′70 | 21705 | 22905 |
| E11 / E′11 | 22295 | 23495 | E71 / E′71 | 21695 | 22895 |
| E12 / E′12 | 22285 | 23485 | E72 / E′72 | 21685 | 22885 |
| E13 / E′13 | 22275 | 23475 | E73 / E′73 | 21675 | 22875 |
| E14 / E′14 | 22265 | 23465 | E74 / E′74 | 21665 | 22865 |
| E15 / E′15 | 22255 | 23455 | E75 / E′75 | 21655 | 22855 |
| E16 / E′16 | 22245 | 23445 | E76 / E′76 | 21645 | 22845 |
| E17 / E′17 | 22235 | 23435 | E77 / E′77 | 21635 | 22835 |
| E18 / E′18 | 22225 | 23425 | E78 / E′78 | 21625 | 22825 |
| E19 / E′19 | 22215 | 23415 | E79 / E′79 | 21615 | 22815 |
| E20 / E′20 | 22205 | 23405 | E80 / E′80 | 21605 | 22805 |
| E21 / E′21 | 22195 | 23395 | E81 / E′81 | 21595 | 22795 |
| E22 / E′22 | 22185 | 23385 | E82 / E′82 | 21585 | 22785 |
| E23 / E′23 | 22175 | 23375 | E83 / E′83 | 21575 | 22775 |
| E24 / E′24 | 22165 | 23365 | E84 / E′84 | 21565 | 22765 |
| E25 / E′25 | 22155 | 23355 | E85 / E′85 | 21555 | 22755 |
| E26 / E′26 | 22145 | 23345 | E86 / E′86 | 21545 | 22745 |
| E27 / E′27 | 22135 | 23335 | E87 / E′87 | 21535 | 22735 |
| E28 / E′28 | 22125 | 23325 | E88 / E′88 | 21525 | 22725 |
| E29 / E′29 | 22115 | 23315 | E89 / E′89 | 21515 | 22715 |
| E30 / E′30 | 22105 | 23305 | E90 / E′90 | 21505 | 22705 |
| E31 / E′31 | 22095 | 23295 | E91 / E′91 | 21495 | 22695 |
| E32 / E′32 | 22085 | 23285 | E92 / E′92 | 21485 | 22685 |
| E33 / E′33 | 22075 | 23275 | E93 / E′93 | 21475 | 22675 |
| E34 / E′34 | 22065 | 23265 | E94 / E′94 | 21465 | 22665 |
| E35 / E′35 | 22055 | 23255 | E95 / E′95 | 21455 | 22655 |
| E36 / E′36 | 22045 | 23245 | E96 / E′96 | 21445 | 22645 |
| E37 / E′37 | 22035 | 23235 | E97 / E′97 | 21435 | 22635 |
| E38 / E′38 | 22025 | 23225 | E98 / E′98 | 21425 | 22625 |
| E39 / E′39 | 22015 | 23215 | E99 / E′99 | 21415 | 22615 |
| E40 / E′40 | 22005 | 23205 | E100 / E′100 | 21405 | 22605 |
| E41 / E′41 | 21995 | 23195 | E101 / E′101 | 21395 | 22595 |
| E42 / E′42 | 21985 | 23185 | E102 / E′102 | 21385 | 22585 |
| E43 / E′43 | 21975 | 23175 | E103 / E′103 | 21375 | 22575 |
| E44 / E′44 | 21965 | 23165 | E104 / E′104 | 21365 | 22565 |
| E45 / E′45 | 21955 | 23155 | E105 / E′105 | 21355 | 22555 |
| E46 / E′46 | 21945 | 23145 | E106 / E′106 | 21345 | 22545 |
| E47 / E′47 | 21935 | 23135 | E107 / E′107 | 21335 | 22535 |
| E48 / E′48 | 21925 | 23125 | E108 / E′108 | 21325 | 22525 |
| E49 / E′49 | 21915 | 23115 | E109 / E′109 | 21315 | 22515 |
| E50 / E′50 | 21905 | 23105 | E110 / E′110 | 21305 | 22505 |
| E51 / E′51 | 21895 | 23095 | E111 / E′111 | 21295 | 22495 |
| E52 / E′52 | 21885 | 23085 | E112 / E′112 | 21285 | 22485 |
| E53 / E′53 | 21875 | 23075 | E113 / E′113 | 21275 | 22475 |
| E54 / E′54 | 21865 | 23065 | E114 / E′114 | 21265 | 22465 |
| E55 / E′55 | 21855 | 23055 | E115 / E′115 | 21255 | 22455 |
| E56 / E′56 | 21845 | 23045 | E116 / E′116 | 21245 | 22445 |
| E57 / E′57 | 21835 | 23035 | E117 / E′117 | 21235 | 22435 |
| E58 / E′58 | 21825 | 23025 | E118 / E′118 | 21225 | 22425 |
| E59 / E′59 | 21815 | 23015 | E119 / E′119 | 21215 | 22415 |
| E60 / E′60 | 21805 | 23005 | E120 / E′120 | 21205 | 22405 |